Smart AI Doorbell

Nyle Farhan Ul Haq and Shivank Chhaya

Problem Statement:

Develop a Raspberry Pi smart doorbell that records visitors, displays a live camera feed, recognizes registered residents, plays a doorbell chime, and demonstrates automatic opening with a servo-driven cardboard door.

Materials: Raspberry Pi 3 Model B+, push button, breadboard, jumper wires, OV5647 camera module, servo motor, cardboard prototype, Bluetooth AirPods, power supply, computer (keypad optional).

Documentation (use single-cell tables)

Set up the wiring and components like this:

Connect button between GPIO17 (physical pin 11) and GND. Connect servo signal to GPIO18 (physical pin 12); use suitable servo power and a shared ground. Attach the camera ribbon cable to the Pi camera port. [Insert wiring photograph/diagram here.]

Illustrative wiring photograph (not a pin-accurate schematic). The keypad shown is not connected in the current program; verify all connections against the written GPIO instructions before wiring.

1. Install necessary packages:

Update package lists and install software for the camera, web server, face recognition, and GPIO.

$ sudo apt update $ sudo apt upgrade -y $ sudo apt install python3-flask python3-opencv opencv-data python3-picamera2 python3-gpiozero -y

2. Create the proper directory and environment:

Organize all scripts and photos in one project folder.

$ mkdir -p ~/smart-doorbell $ cd ~/smart-doorbell $ nano doorbell.py

Paste this code in:

$ [Link to doorbell.py Python file — add later]

3. Test the push button:

Run the initial GPIO17 doorbell script.

$ cd ~/smart-doorbell $ python3 doorbell.py

4. Check the camera:

Confirm that the Pi detects the OV5647 module and can take a photograph.

$ rpicam-hello --list-cameras $ rpicam-still -o test.jpg --timeout 1000

5. Make a visitor photo folder:

Store visitor snapshots separately from source code.

$ cd ~/smart-doorbell $ mkdir -p photos $ ls

6. Create the first Flask dashboard:

Build and start a simple local web server.

$ cd ~/smart-doorbell $ nano app.py $ python3 app.py

Paste this code in:

$ [Link to app.py Python file — add later]

7. Test face detection:

Use the Haar Cascade classifier to find faces in the camera feed.

$ cd ~/smart-doorbell $ nano face_test.py $ python3 face_test.py

Paste this code in:

$ [Link to face_test.py Python file — add later]

8. Verify OpenCV recognition support:

Confirm that the installed OpenCV build supports LBPH.

$ python3 -c "import cv2; print(cv2.__version__)" $ python3 -c "import cv2; print(cv2.face.LBPHFaceRecognizer_create())"

9. Collect resident training photos:

Create two resident folders and run the image collection program. Eventually we collected 100 images per resident.

$ cd ~/smart-doorbell $ mkdir -p faces/resident1 faces/resident2 $ nano collect_faces.py $ python3 collect_faces.py resident1 $ python3 collect_faces.py resident2 $ ls faces/resident1 ¦ wc -l $ ls faces/resident2 ¦ wc -l

Paste this code in:

$ [Link to collect_faces.py Python file — add later]

10. Train the face model:

Train an LBPH model and save it as face_model.yml.

$ cd ~/smart-doorbell $ nano train_faces.py $ python3 train_faces.py

Paste this code in:

$ [Link to train_faces.py Python file — add later]

11. Test resident recognition:

Check whether the camera distinguishes residents from unknown visitors. We used a recognition distance threshold of 55.

$ cd ~/smart-doorbell $ nano recognize_test.py $ python3 recognize_test.py

Paste this code in:

$ [Link to recognize_test.py Python file — add later]

12. Back up and retrain:

Save the old model before rebuilding it with additional photos.

$ cd ~/smart-doorbell $ cp face_model.yml face_model_OLD.yml $ python3 train_faces.py

13. Test the servo motor:

Verify servo rotation for the cardboard door demonstration.

$ cd ~/smart-doorbell $ nano servo_test.py $ python3 servo_test.py

Paste this code in:

$ [Link to servo_test.py Python file — add later]

14. Combine recognition and servo:

Test automatic movement after recognizing an authorized resident. The final system uses five consecutive matching frames.

$ cd ~/smart-doorbell $ nano recognize_servo_WORKING.py $ python3 recognize_servo_WORKING.py

Paste this code in:

$ [Link to recognize_servo_WORKING.py Python file — add later]

15. Install Flask-SocketIO:

Enable instant visitor updates without refreshing every existing photo.

$ sudo apt update $ sudo apt install python3-flask-socketio -y $ python3 -c "import flask_socketio; print('Flask-SocketIO ready')"

16. Create the integrated application:

Combine camera, recognition, button, servo, saved photographs, and dashboard.

$ cd ~/smart-doorbell $ nano smart_system.py $ python3 smart_system.py

Paste this code in:

$ [Link to smart_system.py Python file — add later]

17. Test the dashboard endpoints:

In a separate SSH session, check that the server and visitor history respond.

$ ssh pi@raspberrypi.local $ curl -I http://localhost:5000/ $ curl http://localhost:5000/visitor_history

18. Connect real-time visitor updates:

The dashboard loads existing history once, then adds new photos when it receives the Socket.IO new_visitor event.

$ cd ~/smart-doorbell $ python3 smart_system.py

19. Configure Bluetooth audio:

Set the connected AirPods as the audio output and play a test sound.

$ wpctl status $ wpctl set-default bluez_output.EC_46_54_27_C4_B1.1 $ pw-play /usr/share/sounds/freedesktop/stereo/complete.oga

20. Transfer and test a custom chime:

Copy the custom ding-dong WAV from Windows PowerShell, then test it on the Pi.

$ scp "$env:USERPROFILE\Downloads\doorbell.wav" pi@raspberrypi.local:/home/pi/smart-doorbell/ $ cd ~/smart-doorbell $ ls -l doorbell.wav $ pw-play doorbell.wav

21. Update the chime file in the main program:

Point DOORBELL_SOUND to ~/smart-doorbell/doorbell.wav. Restart the integrated application.

$ cd ~/smart-doorbell $ nano smart_system.py $ python3 smart_system.py

22. Troubleshoot a busy camera:

Stop duplicate programs if the camera reports that it is already in use.

$ ps aux ¦ grep -E "smart_system¦app_WORKING¦python¦rpicam¦libcamera" ¦ grep -v grep $ pkill -f smart_system.py $ pkill -f app_WORKING_BACKUP.py $ rpicam-still -o ~/camera_test.jpg --timeout 1000 $ cd ~/smart-doorbell $ python3 smart_system.py

23. Monitor Raspberry Pi performance:

Check memory, CPU usage, and throttling when the camera stream becomes slow.

$ free -h $ top $ vcgencmd get_throttled

Final result: Pressing the button plays a ding-dong and saves a visitor photo. Socket.IO updates the visitor history in the browser. The camera displays live video and recognizes registered residents; the servo demonstrates opening the cardboard door after five consecutive matches.

Note: Commands are reconstructed from our development work and organized for readability. Some installation commands are consolidated. Add links to the actual Python files and wiring photos before submission.